Rapid processing AMOR images

Michael Knappertsbusch, 12.1.2017 – 26.1.2018, modified 23.10.2019, 19.02.2021

For rapid system file manipulation (copy, paste, delete, etc) on PC see further below.

There are 5 methods to process AMOR derived images:

- Manual processing using Nih Image 1.62b7 and Photoshop on iMac

- Semi-automated processing using Nih Image 1.62b7 and Automation macro on iMac

- Batch processing using Photoshop macro FORAMS on NMB PC

- Semi-automated processing using ImageJ and macro on PC

- Batch processing with AVM on Laptop PC

- Displying correct format of Macintosh derived texts or symbols in Ws Word on a PC/Windows

For the study at ODP Hole 806C the method d (Semi-automated processing using ImageJ and macro on PC) has been applied.

- Manual processing using Nih Image 1.62b7 and Photoshop on iMac:

No batch-processing. This method is very tedious when many images are to be processed.

- Start Nih Image 1.627b7 on iMac.

- Open all grey level images.

- Process/Plugin filters/Invert --> Invert image (necessary because in Photoshop specimen is otherwise black instead of white).

- Use Options/Density slice, adjust until specimen is completely filled with red.

- Process/Binary --> makes a black/white image

- Save as Tiff file

- Start Photoshop

- Open all black/white images

- Save them as raw file (corresponds to a bmp image on a PC).

- Semi-automated processing using Nih Image 1.62b7 and Automation macro on iMac:

Uses „Automation“ macro from Kevin Brown (see chapter 2 in Brown 2007). Faster than first method, but no complete batch processing.

- Start Nih Image on iMac

- Open all grey level images

- Load macro Automation

- Use key p to manually process tob lack/white images

- Use key r to save raw files

- Check raw files in Adobe Photoshop

- Method using Photoshop macro FORAMS on NMB PC:

Complete batch processing, needs NMB PC, Laptop, and iMac. Complicate file handling due to repeated copying from one computer to the next.

- On PC NMB: Copy AMOR Tiff files (grey-level) to a separate folder under Computer/OS (C:)/Benutzer/Öffentlich. Note: If done in a folder on the desktop, this processing is very slow and will take about 30 minutes for 60 images (because loading each image to in Adobe Photoshop CS over the internal network is very slow). If, however, processing is done in OS (C:)/Benutzer/Öffentlich, it runs quite fast (about a second per image).

- Start Photoshop. Process images with macro FORAMS -> Black/white images in Tiff format.

- In ImageJ. Check quality of Black/white images (Tiff). Compare them with original AMOR images. Open ImageJ in a separate explorer (right click on ImageJ icon to open a second ImageJ window), load AMOR images from archive to a stack. Then change to folder on desktop with Black/white images and open images individually. Visually compare whether B/W image was produced correctly from grey level image (click on lower arrow move to next grey level image within stack). If corrections are needed, edit image in first ImageJ window from desktop (use pipette to select color, use polygon to select roi, use Edit/Fill to correct). Save corrected image as xxxx_new into Black/White folder on desktop. After all images are checked rename edited files, delete old ones.

- In ImageJ: Convert to binary of checked & corrected Black/images using macro „converttobinary4“.

- Copy binary images to Laptop.

- On Laptop rename binary images to xxxr format for Mac (no extensions). Use AntRenamer for rapid batch renaming. Note: binary images appear as black images when opened with IrfanView on Laptop. This is so because they became encoded for Macintosh in the above treatement. On iMac they appear in Photoshop as Black/white image in raw format, ready to be processed with the Tracing program.

- Copy binary xxxr images plus necessary files (list_of_files_corr) to iMac.

- On iMac: Process images to outline coordinates with Trace_AMOR1_batch.out. If images needed a correction because of the embayment problem, edit image on iMac with Photoshop, save as raw format.

- Saving files (for later efficient mass storage on PC platforms): Keep final black/white images in raw format (copy also a set tot he Laptop for safety and later composition of image archive). Keep also –bin files from the PC (rename edited xxxr files from iMac if necessary; raw files will later be stored on PC with file extensions and with the names generated by the convertobinary4 macro !!

- Semi-automated processing using ImageJ and macro on PC:

ImageJ 1.48v from Wayne Rasband, National Institute of Health, USA. http://imageJ.nih.gov./ij, Java 1.6.0_20 (32 bit), 3681K of 1638MB.

[Applied for samples ODP Site 806C]

Not as fast as macro forams in Photoshop, but this method allows precise control on thresholding for conversion to black and white images, and runs also on a PC. It is quite efficient.

- Load all Tiff images to stack (File/Import/Image sequence, check sort images numerically).

- Select all (=ctrl a), Smooth, sharpen,smooth,sharpen,smooth (under Process)

- Multiply x 1.6 (sometimes x 1.25) (under Process)

- Save as Stack x.Tiff

- In Stack x scroll through stack, check images, edit where necessary (editing only individual images not batch editing in stack; ctrl A = Select entire stack, ctrl S = Save; ctrl F = Fill; use wand tool (Mode = 8 connected, Tolerance=50). If necessary, use pipette to choose white or black color for filling in individual images.

- Eventuall apply Brightness/Contrast through entire stack (Image/Adjust/Brightness contrast).

- Edit individual images (where necessary) using Wand tool, then cut (makes the particle black), then invert (makes particle white); and/or wand tool/Edit selection/Make inverse/ then cut (makes area outside particle black).

Useful commands: Fill, make inverse, Clear outside, selection/inverse, Edit/selection/enlarge to enlarge or shrink selection. Image/Transform/Rotate…

- Save stack

- Select all, Image/Adjust/Threshold, use glider to select particle best, Then press Apply (Method=Default, Background=dark, all other checkboxes empty). This makes all images in stack binary. Leave little Threshold Panel open.

- Select all(ctrl a), Process/Binary/Close (to fill „one pixel holes“ in outline).

- Save stack

- Once again check images in stack if minor black and white edits are still necessary (if necessary, remove black spots on left side of foram). Save.

- File/Save as/Image sequence, Tiff format (Start at 0, Digits (1-8) = 4, check box to keep file names). This overwrites all existing grey level images with black/while images in Tiff format.

- File/Save as/Image sequence, Raw format (check box to keep file names). This saves all black/while images in Raw format, so that they can be processed with Trace program on Mac.

- No longer necessary - [Load Yannick Mary’s macro „Converttobinary_corrected4_raw“ converts all Tiff images to raw files (keeping the file name), so that they can be processed with Trace program on Mac].

- Organize Tiff and raw files into separate folders B_W_Tiff and Raw files, respectively.

Move List_of_files.txt and List_of_files_corr.txt into Raw files.

- Copy directories B_W_Tiff and Raw files to respective directory on Archive (N-server).

- Copy raw images to Mac, rename files using program Rename for input into Trace program.

- Alternatively: Copy files to Laptop to convert filenames with AntRenamer. Copy renamed files to Mac.

- On Mac - Outline extraction with Trace program.

- Batch processing with AVM on Laptop PC:

Complete batch processing, produces no saved raw files but directly traced (_T) files, determination of DeltaX distance in results file. AVM version 1.0 from S. Dietiker produces outline coordinates in pixels only. AVM version 1.5 (=modification by Strelcenko) produces outline coordinates in micrometers, however X and Y extractions are not correct (this version uses oblique values of X, which is false).

Runs completely on Laptop and is very fast, processing to outline can be visually observed while running.

Since . . .we have improved version AVM 2.0 (=modification by Jean Eisenecker), where dX values are corrected. AVM2.0 version not yet thoroughly tested (23.10.2019).

Rapid file manipulation (copy, paste, delete, etc) on PC:

For example delete all FINDER.DAT files after copying many folders from Mac to PC:

- Leftclick on and activate highest directory where all files are in.

- In Windows Explorer: Go to search tool (mit Lupe, im Fenster ganz oben rechts), enter FINDER.DAT

Then Windows Explorer lists all FINDER.DAT files in all subdirectories.

- To delete them all at once: Mouseclick and activate them, then right click, select delete. All FINDER.DAT files in all subdirectories will so be deleted.

- Repeat above steps for other Macintosh system files like RESOURCE.FRK, or Thumbs.db.

Rapid renaming files on PC:

With program Ant Renamer 2.10:

Open Ant Renamer.

Click Add files  Box opens, mouse-select all files to rename, click open.

In case that all files to be renamed are distributed in directories and subdirectories, then check boxes in the following manner:

Select option «Add files of selected folders»

De-select option «Add selected folders»

Select option «Apply these two options also to subfolders»

To add .txt extension to file names go to Menu Actions: select Change extension; select Replace extension by .txt; check box «Do not replace extension, append it to current file name».

Click to Files in the menu (to see which files will be changed).

Then click to Go in the menu  it appends the .txt extension to every file.

At the end do not forget to click Files (in the menu) and to click Remove all ( it removes the files from the window; otherwise it will add the appendix .txt to files once again, which results to .txt.txt in the next run with new files).

Note on displying correct format of Macintosh derived texts or symbols in Ms Word on a PC/Windows:

When opening Mac-derived text files (save as .txt) but in Microsoft Word and on a PC, select «andere Codierung: Westeuropäisch (Mac)» in order to correctly display symbols of  or . If only Windows (Standard) is checked, special characters like  or  may sometimes (not always) appear as cryptic characters.